Министерство образования Республики Беларусь

Учреждение образования

БЕЛОРУССКИЙ ГОСУДАРСТВЕННЫЙ УНИВЕРСИТЕТ

ИНФОРМАТИКИ И РАДИОЭЛЕКТРОНИКИ

Факультет компьютерных систем и сетей

Кафедра электронных вычислительных машин

ОТЧЕТ

по лабораторной работе №5

Вариант №212

Студент: 								       П. Е. Казаченко

Проверил:							                 В. А. Марцинкевич

МИНСК 2025

1. Практическая часть

1.1. Топология

1.2. Рабочие конфигурации

R1:

ipv6 unicast-routing

interface f0/0

ipv6 address fd00:1::2/64

ipv6 nd reachable-time 60000

no shutdown

exit

interface Loopback0

ipv6 address fd00:12::1/64

ipv6 address 2a00:1760:52:8000::1/128

exit

ipv6 route ::/0 fd00:1::1

R2:

ipv6 unicast-routing

interface f1/1

ipv6 address fd00:6::2/64

no shutdown

exit

interface f1/0

ipv6 address fd00:4::1/64

no shutdown

exit

interface f0/0

ipv6 address fd00:3::1/64

no shutdown

exit

ipv6 route ::/0 fd00:6::1

R3:

ipv6 unicast-routing

interface f0/0

ipv6 address fd00:9::1/64

no shutdown

exit

interface f1/0

ipv6 address fd00:7::1/64

no shutdown

exit

interface f1/1

ipv6 address fd00:10::2/64

no shutdown

exit

interface Loopback0

ipv6 address fd00:13::1/64

ipv6 address 2a00:1760:51::1/128

exit

ipv6 route ::/0 fd00:7::2

R4:

ipv6 unicast-routing

interface f0/0

ip address 173.102.66.1 255.255.255.0

no shutdown

exit

interface f1/0

ipv6 address fd00:4::2/64

no shutdown

exit

interface f1/1

ipv6 address fd00:5::2/64

no shutdown

exit

interface Tunnel0

ipv6 address fd00:2::1/64

tunnel source 173.102.66.1

tunnel destination 173.102.66.2

tunnel mode ipv6ip

no shutdown

exit

ipv6 route ::/0 fd00:2::2

R5:

ipv6 unicast-routing

interface f0/0

ipv6 address fd00:6::1/64

no shutdown

exit

interface f1/0

ipv6 address fd00:9::2/64

no shutdown

exit

interface f1/1

ipv6 address fd00:11::1/64

no shutdown

exit

ipv6 route ::/0 fd00:11::2

R6:

ipv6 unicast-routing

interface f0/0

ipv6 address fd00:5::1/64

no shutdown

exit

interface f1/0

ipv6 address fd00:8::2/64

no shutdown

exit

interface f1/1

ipv6 address fd00:7::2/64

no shutdown

exit

ipv6 route ::/0 fd00:5::2

R7:

ipv6 unicast-routing

interface f0/0

ipv6 address fd00:1::1/64

no shutdown

exit

interface f1/1

ipv6 address fd00:3::2/64

no shutdown

exit

interface f1/0

ip address 173.102.66.2 255.255.255.0

no shutdown

exit

interface Tunnel0

ipv6 address fd00:2::2/64

tunnel source 173.102.66.2

tunnel destination 173.102.66.1

tunnel mode ipv6ip

no shutdown

exit

ipv6 route ::/0 fd00:3::1

ipv6 route fd00:1::/64 fd00:8::2

R8:

ipv6 unicast-routing

interface f0/0

ipv6 address fd00:8::1/64

no shutdown

exit

interface f1/0

ipv6 address fd00:11::2/64

no shutdown

exit

interface f1/1

ipv6 address fd00:10::1/64

no shutdown

exit

ipv6 route ::/0 fd00:10::2