Министерство образования Республики Беларусь

Учреждение образования

«Белорусский государственный университет информатики и радиоэлектроники»

Кафедра электронных вычислительных машин

ОТЧЕТ

по лабораторной работе №2

Вариант № 212

Выполнил                                                                               Проверил:

студент группы 250504:                                                        Марцинкевич В.А.

Казаченко П.Е.

Минск 2025

Цель работы: сконфигурировать VLAN и настроить STP в топологии.

Рабочие конфигурации устройств:

all switches:

vlan 10

vlan 20

vlan 99

vlan 100

spanning-tree vlan 1,10,20,99,100

N3:

int fa0/4

switchport mode access

switchport access vlan 100

switchport port-security

switchport port-security mac-address 0001.C766.CE17

int fa0/5

switchport mode access

switchport access vlan 20

int fa0/6

switchport mode access

switchport access vlan 10

int range fa0/4-6

spanning-tree bpduguard enable

spanning-tree portfast

int fa0/1-3

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shut

N1:

int range fa0/2-3

switchport mode access

switchport access vlan 10

int fa0/4

switchport mode access

switchport access vlan 20

int range fa0/2-4

spanning-tree bpduguard enable

spanning-tree portfast

int fa0/1

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shut

N6:

int range fa0/1-4

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

int range fa0/1,fa0/4

switchport mode trunk

channel-group 1 mode on

no shutdown

int port-channel 1

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shutdown

N2:

int range fa0/1-4

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

int range fa0/2,fa0/4

switchport mode trunk

channel-group 2 mode on

no shutdown

int port-channel 2

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shutdown

N7:

int range fa0/1-4

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

int range fa0/3,fa0/4

switchport mode trunk

channel-group 3 mode on

no shutdown

int port-channel 3

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shutdown

N4:

spanning-tree vlan 10 priority 4096

spanning-tree vlan 20 priority 4096

spanning-tree vlan 99 priority 4096

spanning-tree vlan 100 priority 4096

default interface fa0/1

default interface fa0/2

int range fa0/1-2

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

channel-group 1 mode on

no shutdown

int port-channel 1

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shutdown

default interface fa0/3

default interface fa0/4

int range fa0/3-4

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

channel-group 2 mode on

no shutdown

int port-channel 2

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shutdown

default interface fa0/5

default interface fa0/6

int range fa0/5-6

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

channel-group 3 mode on

no shutdown

int port-channel 3

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shutdown

int fa0/7

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

no shutdown

R1:

spanning-tree vlan 1,10,20,99,100

int fa0/0.10

encapsulation dot1q 10

ip address 192.168.10.1 255.255.255.0

no shut

int fa0/0.20

encapsulation dot1q 20

ip address 192.168.20.1 255.255.255.0

no shut

int fa0/0.100

encapsulation dot1q 100

ip address 192.168.100.1 255.255.255.0

no shut

int fa0/0

no shutdown

N5:

int fa0/1-3

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

N8:

int fa0/1-3

switchport mode trunk

switchport trunk allowed vlan 1,10,20,99,100

switchport trunk native vlan 99

PC0:

IP: 192.168.100.2

MASK: 255.255.255.0

GATE: 192.168.100.1

PC1:

IP: 192.168.20.2

MASK: 255.255.255.0

GATE: 192.168.20.1

PC2:

IP: 192.168.10.2

MASK: 255.255.255.0

GATE: 192.168.10.1

PC3:

IP: 192.168.20.3

MASK: 255.255.255.0

GATE: 192.168.20.1

PC4:

IP: 192.168.10.4

MASK: 255.255.255.0

GATE: 192.168.10.1

PC5:

IP: 192.168.10.3

MASK: 255.255.255.0

GATE: 192.168.10.1

Практическая часть (скриншот реализованной топологии):

Вывод: В ходе работы все коммутаторы были настроены на работу с vlan. Пользовательские станции по обе стороны топологии были определены к vlan10 и vlan20, одно из устройств расположилось в админском vlan100, vlan99 был выделен как native vlan для всех trunk-соединений. На коммутаторах по краям топологии были настроены portfast, bdpuguard и port-security.

Рисунок 1 – Результат работы